www.sunjinlab.com

Product Information

SunJin Lab

Optical Clearing Innovation

June 2016

Rap i Clear ® CS

CLARITYTM Specific mounting reagent (RI=1.45nD)

RapiClear® CS mounting solution RapiClear® CS mounting gel

RapiClear®  CS  is  an aqueous-based clearing reagent with the   same   refractive   index   as   the   CLARITYTM     (Stanford university)  hydrogel–tissue  hybrid  (1.45nD).  Lipid-extracted hydrogel   effectively   becomes    uniformly   transparent   after immersion  in  RapiClear®  CS  mounting  solution.  As  a  result, sample image quality can reach to cellular resolution even at the depth of several millimeters using confocal microscopy or light sheet illumination microscopy.

RapiClear® CS mounting solution (1.45nD) is a liquid and ready to use reagent. After SDS was completely washed out by incubating the hydrogel in PBS and PBST solution, hydrogel can be directly transferred  into mounting  solution  for  optical clearing.

RapiClear® CS mounting gel (1.45nD) is a jelly-like reagent

。

with the melting temperature in approximate 75 C and gelling

。

temperature below 30 C. It is designed for mounting hydrogel sample  cleared  with  the  RapiClear®   CS  mounting   solution. RapiClear® CS mounting gel does not affect the clearing effect of mounting  solution.  Samples  mount   in  mounting  gel  can prevent hydrogel from swelling as well as floating.

# OPERATION

1.    Hydrogel–tissue  hybrid   (sample)  cleared  with   8%  SDS solution  and  then  washed  with  0.2%  PBST  and  PBS thoroughly.

2.    Transfer  sample  to  a  well  plate  or  centrifuge  tube  with RapiClear® CS mounting solution 5~ 10 times of the sample volume.  Sample  will  regain  the  original  size  within  24 hours.

3.    Sample  is  then  directly  embedded  to  the  RapiClear®  CS mounting gel. Preparation of embedded sample requires a container suitable for confocal observation.   The  simplest approach is to use iSpacer (SunJin Lab Co.).

3-1  Place  the  mounting  gel  in  hot-water  bath  or  heating

。

block (75 C) for 15 min.

。

3-2  Once molten, the mounting gel is left to cool to 40 C in a water bath or on a heating plate. It is very important

。

to ensure that the mounting gel is at 40 C before use.

3-3  Users can aliquot mounting gel to 2~5 ml tubes for later

。

use.  Label and store at 4 C.

3-4  Sample is introduced to the container with blunt-ended forceps. iSpacer with 24 x 40 mm coverslip is used as a container, www.sunjinlab.com/demo.

3-5  Place the molten mounting gel to the container. Avoid bubble formation during handling.

3-6  Place a 24 x 40 mm coverslip and seal with nail polish. Avoid bubble formation during handling.

3-7   Let the mounting gel polymerize at RT for 30 minto  1 hour.

4.    Confocal imaging.

5.    Detailed   CLARITYTM     protocol   is   available   online   at

http://capture-clarity.org/

# REFERENCE

Ye, L.#, Allen, W.E.#, Thompson, K.R.#, Tian, Q., Hsueh, B.,  Ramakrishnan,  C., Wang, A.C.,  Jennings,  J.H., Adhikari, A.,  Halpern, C.H., Witten, I.B., Barth, A.L., Luo, L., McNab, J.A.,  Deisseroth,  K.*  (2016).  Wiring  and  molecular  features  of prefrontal ensembles representing distinct experiences. Cell 165:

1-13. *corresponding authors, # equal contribution. doi: http://dx.doi.org/10.1016/j.cell.2016.05.010

# STABILITY AND STORAGE

RapiClear®  CS can be stored at -20℃~RT. When stored at  ≤ 4℃, the product is stable for at least  12 months.

# WARNING AND PRECAUTIONS

Repeated exposure may cause skin dryness or cracking. Prevent skin contact is suggested.

# TECHNICAL ASSITANCE

E-mail: daniel@sunjinlab.com

Rap i Clea r ® CS

Manufacturer:

SunJin Lab Co.

www.sunjinlab.com

4F., No. 11, Ln. 22, Bo’ai St., East Dist., Hsinchu City 30068, Taiwan

Phone: 886-3-516-5085 Fax: 886-3-516-5087